Run following commands on all routers

Enable secret tyit1819

Router ospf 1

area 0 authentication message-digest

exit

Configure message digest

Configure the MD5 key for all the routers in area 0.

R1(config)# interface s0/0/0

R1(config-if)#ipospf message-digest-key 1 md5 MD5pa55

R2(config)# interface s0/0/0

R2(config-if)#ipospf message-digest-key 1 md5 MD5pa55

R2(config-if)# interface s0/0/1

R2(config-if)#ipospf message-digest-key 1 md5 MD5pa55

R3(config)# interface s0/0/1

R3(config-if)#ipospf message-digest-key 1 md5 MD5pa55

Configure Local AAA Authentication for Console Access on R

- Test connectivity PCA to PCB and PCC ,PCB to PCC

- Configure a local username on R1.

R1(config)# username Admin1 secret admin1pa55

- Configure local AAA authentication for console access on R1.

R1(config)# aaa new-model

R1(config)# aaa authentication login default local

- Configure the line console to use the defined AAA authentication method.

R1(config)# line console 0

R1(config-line)#login authentication default

- Verify the AAA authentication method.

R1(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R1# exit

Configure Local AAA Authentication for vty Lines on R1

- Configure domain name and crypto key for use with SSH.

R1(config)# ip domain-name ccnasecurity.com

R1(config)# crypto key generate rsa

- Configure a named list AAA authentication method for the vty lines on R1.

R1(config)# aaa authentication login SSH-LOGIN local

- Configure the vty lines to use the defined AAA authentication method.

R1(config)# line vty 0 4

R1(config-line)#login authentication SSH-LOGIN

R1(config-line)#transport input ssh

R1(config-line)# end

- Verify the AAA authentication method.

PC-A..

PC>ssh –l Admin1 192.168.1.1

Configure Server-Based AAA Authentication Using TACACS+ on R2

Step 1: Configure a backup local database entry called Admin.

R2(config)# username Admin2 secret admin2pa55

Step 2: Verify the TACACS+ Server configuration

Click the TACACS+ Server. On the Services tab, click AAA. Notice that there is a Network configuration entry for R2 and a User Setup entry for Admin2.

Step 3: Configure the TACACS+ server specifics on R2.

Configure the AAA TACACS server IP address and secret key on R2.

R2(config)# tacacs-server host 192.168.2.2

R2(config)# tacacs-server key tacacspa55

Step 4: Configure AAA login authentication for console access on R2.

R2(config)# aaa new-model

R2(config)# aaa authentication login default group tacacs+ local

Step 5: Configure the line console to use the defined AAA authentication method.

R2(config)# line console 0

R2(config-line)# login authentication default

R1(config-line)#do wr

Step 6: Verify the AAA authentication method.

Configure Server-Based AAA Authentication Using RADIUS on R3

Step 1: Configure a backup local database entry called Admin.

R3(config)# username Admin3 secret admin3pa55

Step 2: Verify the RADIUS Server configuration.

Click the RADIUS Server. On the Services tab, click AAA. Notice that there is a Network configuration entry for R3 and a User Setup entry for Admin3.

Step 3: Configure the RADIUS server specifics on R3.

Configure the AAA RADIUS server IP address and secret key on R3.

R3(config)# radius-server host 192.168.3.2 R3(config)# radius-server key radiuspa55

Step 4: Configure AAA login authentication for console access on R3.

R3(config)# aaa new-model

R3(config)# aaa authentication login default group radius local

Step 5: Configure the line console to use the defined AAA authentication method.

R3(config)# line console 0 Configure AAA Authentication on Cisco Routers R3(config-line)# login authentication default

R3(config-line)#do wr

Step 6: Verify the AAA authentication method.